# On Calibrating a Projector-Camera Rig to Enable Spatial Augmented Reality Use Cases Using cpsspatialar

Michael Hornacek

TU Vienna (IMW-CPS)

May 2021

# Motivation

Spatial augmented reality (SAR) is a branch of augmented reality whereby the augmentation of the scene is carried out using a projector. While pointing a projector at a scene and projecting an image is on its own not a challenge, doing so in a manner such that the projected content appear to be naturally part of the scene is more tricky. The intuition for the general problem we face can be drawn from using a projector to project an image to a flat wall: unless the projector faces the wall frontally, the bounds of a projected rectangular image will not appear rectangular, but more generally as a trapezoid (i.e., the image will appear distorted). The cause for this effect is that the geometry of the scene as it relates to the geometry of the projector plays a role in how the pixels of the projector’s image plane map to points in the scene.

The central objective we set for ourselves is to enable correcting for distortion caused by projecting to arbitrary scene geometry in an automatic fashion. This is to be done in a manner that lends itself to practical application in a real-world setting where the scene to be augmented—e.g., an object being gradually assembled and its placement relative to the projector—does not remain static. Moreover, the object in question may but may not in its entirety fall within the projector’s field of view (FOV). Finally, the solution is ideally to rely on the existing hardware setup—i.e., projector with steerable mirror system and stereo camera—mounted in the Pilotfabrik of TU Vienna.

# Approach

The problem of correcting for distortions as outlined in Section 1 requires knowledge of the manner in which the respective rays through the pixels of the projector’s image plane fan out into the scene, and the geometry of the scene itself within at least the projector’s field of view (determined in part by the steerable mirror), as illustrated in Figure 1. This is because the scene point ‘illuminated’ by a pixel in the projector’s image plane is given by intersecting its corresponding ray with the geometry of the scene surface. To model this interaction calls for two components in particular: (i) one-time projector calibration and (ii) recovery of scene geometry whenever the scene undergoes change relative to the field of view of the projector.

Figure 1: Projector-scene geometry. The way rays ‘fan out’ through the pixels of the projector’s image plane is given by calibrating the projector (note that the projector’s field of view is the portion of the scene in between the gray rays). The scene point ‘illuminated’ by a pixel in the projector’s image plane is given by intersecting its corresponding ray with the geometry of the scene surface. Projector calibration and recovery of scene geometry are accordingly prerequisites of the image warping that it is our ambition to carry out.

The remainder of this section is organized in accordance with these two components: one-time projector calibration, and recovery of scene geometry.

## One-time Projector Calibration

The ‘output’ of projector calibration is a mathematical model that determines, for each pixel xproj in the projector’s image plane πproj, the ray from the projector’s center of projection Cproj through xproj, along which the pixel is projected to the scene. Inversely, the model also determines the projection x’proj of any point X’proj in the scene to the projector’s image plane πproj, obtained by intersecting πproj with the ray from Cproj through X’proj. Such a model is said to function in accordance with the principle of central projection.

Figure 2: Pinhole camera model. The image plane π is perpendicular to the Z axis of the camera’s coordinate frame; its location is given by the focal length f, the distance of π along the Z axis. The camera’s center of projection C is located at the origin 0 of the camera coordinate frame. The projection x = (xcam, ycam) of a point X = (X, Y, Z) to π, by central projection, follows from the principle of similar triangles. Note that the origin of the coordinate system of the image plane—called the principal point—is situated at the intersection of the Z axis with π. Note that the Z component of a point expressed in the camera’s coordinate frame is referred to as the point’s depth relative to the camera.

Notably, central projection is the same principle that underlies the modelling of conventional cameras. A camera modeled in accordance with the principle of central projection is called a pinhole camera. A pinhole camera—as illustrated in Figure 2—can be expressed in terms of a 3 × 3 ‘camera calibration’ matrix K, derived from the camera’s focal length and principal point. In practice, a pinhole camera model works well in modelling how the respective rays through the pixels of the image plane fan out into the scene, on the condition that images to which the model is applied have been corrected for lens distortions caused by the camera’s lens system (cf. Figure 3 for an example). The ‘output’ of camera calibration is accordingly typically not only the matrix K, but the associated lens distortions model coefficients as well. The lens distortion model coefficients enable applying a lens distortion model to undistort any image acquired by the camera.

In practice, calibrating a camera relies on (i) establishing 2D-3D correspondences between pixels in the camera’s image plane and corresponding points in the scene, and on (ii) using those correspondences as input to an optimization procedure that outputs the calibration matrix K and the associated distortion model coefficients. Calibration of a projector can be carried out in precisely the same manner insofar as step (ii) is concerned; the major difference in projector calibration relative to camera calibration concerns the manner in which 2D-3D correspondences are identified, i.e., between pixels in the image plane of the projector and the corresponding points in the scene.

What remains of this section first reviews how 2D-3D correspondences are conventionally obtained between camera and scene. Next, we address how we proceed to obtain 2D-3D correspondences between projector and scene. Finally, we outline the manner in which 2D-3D correspondences are used to compute the calibration matrix K and associated lens distortion model coefficients. Camera and projector calibration are implemented in cpsspatialar as described in this section, respectively, by the apps calibrateCam (which is capable of calibrating a single camera or a stereo pair) and calibrateProj (which takes a camera calibration produced by calibrateCam as input).

### Obtaining Camera-Scene 2D-3D Correspondences

The recovery of 2D-3D correspondences in support of camera calibration is carried out, conventionally, by relying on a planar calibration surface of known scale to automatically identify correspondences between the 3D points on the calibration surface and their 2D correspondences in the image plane. A widely used example of such a calibration object is a chessboard pattern. The 3D corner points of the chessboard are obtained analytically in a coordinate system defined in the plane of the chessboard, requiring knowledge only of the length of a side of a chessboard square. Next, using the findChessboardCorners() function in OpenCV the corresponding 2D pixels of the corner points in any single image of the chessboard are obtained as a list; an example is given in Figure 3.

In practice, multiple images of the chessboard are acquired, from a set of widely varying viewpoints. The rationale behind doing so is to capture the varying effects of lens distortion across the full extent of the camera’s field of view; notably, images of the chessboard acquired only at approximately the middle of the image plane would fail to provide information about radial distortions present at the image plane’s boundaries.

Figure 3: 2D chessboard corners recovered automatically using OpenCV’s findChessboardCorners() function, color coded using the drawChessboardCorners() function. The corresponding 3D positions can be computed analytically, given prior knowledge of the length of a square of the chessboard. Note the distortions caused by the camera’s lens system, clearly visible in the image: the edges of the chessboard should be straight lines, but instead appear to bend outwards.

### Obtaining Projector-Scene 2D-3D Correspondences

Todo: describe how we are now obtaining 3D points via intersection with ground plane of back-projections of detected circles… Whether we want to project to a ground plane or to arbitrary scene geometry, we will calibrate our projector by projecting circles to the floor. Besides recovering the intrinsics of the projector, for each of the locations on the floor to which we project and thereby for precisely the respective projector look directions, we will obtain the extrinsics of the projector. These will be the projector look directions for which we will be able to carry out spatial AR.

| (a) | (b) |
| --- | --- |

Figure 4: todo... (a) Projected circle pattern, with 2D circle centers obtained using again the function findChessboardCorners(), color coded using drawChessboardCorners(). (b) The corresponding 3D locations on the ground of the circle centers for any image of the circle pattern are obtained by computing the intersection of the ground plane (recovered by camera resection with respect to the known K of the camera—obtained by calibrating the camera in a prior step—and 2D-3D correspondences with respect to the chessboard pattern placed on the floor) with the back-projections of the detected circle centers …

### Computing the Calibration

Given a set of 2D-3D correspondences in the form of a list of lists of 2D pixel positions (‘image points’) and a corresponding list of 3D points (‘object points’), the OpenCV function calibrateCamera() can be used to carry out the actual calibration. The function returns the camera calibration matrix K and the lens distortion model coefficients. Additionally, the function returns a list of 3D rigid body transformations whose length equals the number of calibration images used as input; the nth such rigid body transformation gives the pose (R, t) of the camera or projector relative to the corresponding 3D points as seen in the nth calibration image. The projection of such a 3D point X to the image plane of the camera or projector with respect to the nth calibration image is given by K(RX + t), and is obtained in a manner such that, collectively over all 2D-3D correspondences, the sum of squared distances in the image plane between the 2D projections of these 3D points and their respective 2D correspondence are minimized.

## Computing the Warp

Todo: the discussion of stereo here is relevant to the case where we want to project to arbitrary surface geometry. We will pursue this as out next step (and then texture map the resulting mesh). For our first step (projecting to a flat surface), we get the ground plane using a checkerboard pattern…

### Planar Scene Geometry

Todo… Warp via plane-induced homography with respect to ground plane, viewpoint of virtual projector, and viewpoint of original projector…

### Arbitrary Scene Geometry

Fundamental to all variants of stereo is that, given a pixel x in the left image, the search for the corresponding pixel x’ in the right image is restricted to only pixels situated along the epipolar line spanned by x, as illustrated in Figure 5. Where stereo approaches differ is primarily in how the ‘best’ match along an epipolar line is determined. Classical correlation-based stereo approaches rely on maximizing a similarity score over a window about x in the left image and the corresponding windows about respective candidate matching pixel in the right image. However, a correlation-based approach can be expected to work well only in the presence of adequately discriminative texture. A way around this is to introduce discriminative texture by projecting a pattern to the scene. ... todo

Warp via texture mapping recovered mesh from viewpoint of virtual projector, and rendering resulting textured mesh from viewpoint of original projector…

Figure 5: Epipolar geometry (i.e., the geometry of stereo). The 3D point X that projects to the pixel x in the left camera’s image plane must lie along the ray passing through x from the left camera’s center of projection C. Accordingly, the projection x’ of X to the image plane of the right camera must lie along the epipolar line l’ spanned by x, obtained by projecting that ray to the right camera’s image plane. This restricts the search for x’ to only the pixels along that line, a constraint common to all stereo approaches. Once x’ has been identified, the point X can be recovered by triangulation.

# Building the Code

Let CPSSPATIALAR_BASEDIR refer to the base directory of your local copy the cpsspatialar Git repository (such that src and docs are subdirectories of CPSSPATIALAR_BASEDIR):

- Install Visual Studio Professional 2019

- Install CMake

- Install Git

- Let VCPKG_BASEDIR refer to HOME_DIR/vcpkg; install vcpkg, having cloned from within HOME_DIR (such that vcpkg.exe is then located in VCPKG_BASEDIR)

- Set the environment variable CMAKE_TOOLCHAIN_FILE to point to VCPKG_BASEDIR\scripts\buildsystems\vcpkg.cmake

- Let EIGEN_BASEDIR refer to CPSSPATIALAR_BASEDIR\src\external\eigen; download version 3.4 of the source code of Eigen and extract to EIGEN_BASEDIR (such that the CMakeLists.txt file is located in EIGEN_BASEDIR)

- Let OPENCV_BASEDIR refer to CPSSPATIALAR_BASEDIR\src\external\opencv; download version 4.5.1 of the source code of OpenCV and extract to OPENCV_BASEDIR (such that the CMakeLists.txt file is located in OPENCV_BASEDIR)

- Run CMake (cmake-gui):

- Enter OPENCV_BASEDIR in the field to the right of ‘Where is the source code’ and OPENCV_BASEDIR\build in the field to the right of ‘Where to build the binaries’ (note that backslashes are converted automatically to forward slashes)

- Click ‘Configure’ and click ‘Yes’ if prompted to create the directory OPENCV_BASEDIR

- If prompted to specify the generator for this project and to optionally indicate the platform for the generator, provide ‘Visual Studio 16 2019’ and ‘x64’, respectively, and click ‘Finish’

- Click ‘Open Project’ or, equivalently, open OPENCV_BASEDIR\build\OpenCV.sln; this opens the Visual Studio solution OpenCV.sln in Visual Studio Professional 2019

- Select ‘Release’ from the Solution Configuration pull-down menu and select ‘Build:Build Solution’ from the menu bar to finally build (i.e., compile) OpenCV in release mode (note that it will take some minutes for building to complete):

- Let FREEGLUT_BASEDIR refer to CPSSPATIALAR_BASEDIR\src\external\ freeglut; download version 2.8.1 of freeglut pre-built binaries to FREEGLUT_BASEDIR (such that the CMakeLists.txt file is located in FREEGLUT_BASEDIR)

- Let GLEW_BASEDIR refer to CPSSPATIALAR_BASEDIR\src\external\glew; download version 2.1.0 of the source code of glew and extract to GLEW_BASEDIR (such that the Makefile file is located in GLEW_BASEDIR)

- Run CMake (cmake-gui)

- Enter GLEW_BASEDIR\build\cmake in the field to the right of ‘Where is the source code’ and GLEW_BASEDIR in the field to the right of ‘Where to build the binaries’ (note that backslashes are converted automatically to forward slashes)

- Click ‘Configure’

- If prompted to specify the generator for this project and to optionally indicate the platform for the generator, provide ‘Visual Studio 16 2019’ and ‘x64’, respectively, and click ‘Finish’

- Click ‘Open Project’ or, equivalently, open GLEW_BASEDIR\glew.sln; this opens the Visual Studio solution glew.sln in Visual Studio Professional 2019

- Select ‘Release’ from the Solution Configuration pull-down menu and select ‘Build:Build Solution’ from the menu bar to build glew in release mode

- Let GLFW_BASEDIR refer to CPSSPATIALAR_BASEDIR\src\external\glfw; download version 3.3.4 of the source code of glfw and extract to GLFW_BASEDIR (such that the Makefile file is located in GLFW_BASEDIR)

- Run CMake (cmake-gui)

- Enter GLFW_BASEDIR in the field to the right of ‘Where is the source code’ and GLFW_BASEDIR\build in the field to the right of ‘Where to build the binaries’ (note that backslashes are converted automatically to forward slashes)

- Click ‘Configure’

- If prompted to specify the generator for this project and to optionally indicate the platform for the generator, provide ‘Visual Studio 16 2019’ and ‘x64’, respectively, and click ‘Finish’

- Click ‘Open Project’ or, equivalently, open GLFW_BASEDIR\glfw.sln; this opens the Visual Studio solution glew.sln in Visual Studio Professional 2019

- Select ‘Release’ from the Solution Configuration pull-down menu and select ‘Build:Build Solution’ from the menu bar to build glfw in release mode

- Open the terminal, cd to VCPKG_BASEDIR, and run vcpkg install --triplet=x64-windows pcl followed by vcpkg integrate install to install PCL (Point Cloud Library) and make it discoverable by Visual Studio, respectively

- Open the terminal, cd to VCPKG_BASEDIR, and run vcpkg install --triplet=x64-windows ceres followed by vcpkg integrate install to install Google Ceres and make it discoverable by Visual Studio, respectively

- Run CMake (cmake-gui)

- Enter CPSSPATIALAR_BASEDIR\src in the field to the right of ‘Where is the source code’ and CPSSPATIALAR_BASEDIR\build in the field to the right of ‘Where to build the binaries’ (note that backslashes are converted automatically to forward slashes)

- Click ‘Configure’ and click ‘Yes’ if prompted to create the directory CPSSPATIALAR_BASEDIR

- If prompted to specify the generator for this project and to optionally indicate the platform for the generator, provide ‘Visual Studio 16 2019’ and ‘x64’, respectively, select ‘Specify toolchain file for cross-compiling’, and click ‘Next’

- Enter VCPKG_BASEDIR\scripts\buildsystems\vcpkg.cmake as the path to the toolchain file and click ‘Finish’

- Click ‘Open Project’ or, equivalently, open CPSSPATIALAR_BASEDIR\build\cpspatialar.sln; this opens the Visual Studio solution cpspatialar.sln in Visual Studio Professional 2019

- Select ‘Release’ from the Solution Configuration pull-down menu and select ‘Build:Build Solution’ from the menu bar to build cpsspatialar in release mode

The resulting executables can be found at CPSSPATIALAR_BASEDIR\build\bin\Release

https://www.pilotfabrik.at/?lang=en

Cf. Section 4.1.2 of the Spatial Augmented Reality textbook of Bimber and Raskar, available at http://pages.cs.wisc.edu/~dyer/cs534/papers/SAR.pdf. Note that the authors also mention user location; this is relevant for some—but not all—use cases, and we shall leave it too outside the scope of this document.

Cf. again Section 4.1.2 of Bimber and Raskar.

If you’ll forgive a brazen bit of self-indulgence, have a look at Chapter 2 of my dissertation at https://publik.tuwien.ac.at/files/PubDat_243668.pdf for a more detailed attempt at a self-contained but accessible introduction to the geometry pinhole cameras, the derivation of the camera calibration matrix K, and other topics of relevance to this document (including epipolar geometry).

Cf. https://markhedleyjones.com/projects/calibration-checkerboard-collection for a variety of chessboard calibration patterns ready for use.

E.g., (0,0,0), (1.5,0,0), (3,0,0), …, (9,7.5,0) for a chessboard with 7 × 6 corners, with each square of length and width of 1.5 unit, respectively.

See https://docs.opencv.org/master/dc/dbb/tutorial_py_calibration.html for a comprehensive tutorial.

It is good practice to acquire images of the calibration surface across all corners and edges of the image plane, along with an image or two from around the middle of the image plane. Beware: for any given viewpoint, the entirety of the chessboard must be visible if relying on OpenCV’s findChessboardCorners(). Otherwise, the function will fail to find any corners in the image acquired from that viewpoint.

Each list contains a list corresponding to each respective image.

See again https://docs.opencv.org/master/dc/dbb/tutorial_py_calibration.html for a comprehensive tutorial.

See https://www.learnopencv.com/understanding-lens-distortion/ for more details on how these particular coefficients are interpreted mathematically.

Note that if these points are of a calibration surface placed on the floor, then such a rigid body transformation gives the pose of the projector or camera relative to the ‘ground plane’; the inverse of this rigid body transformation gives the pose of the ground plane relative to the projector or camera.

Such residuals are referred to as reprojection errors in computer vision.

Objects like the white walls of a room or typical manmade objects are too poorly textured to enable reliable matching via correlation-based stereo.

https://github.com/m-hornacek/cpsspatialar

Available via TU Wien Campus Software at https://www.it.tuwien.ac.at/en/services/software/software-provision/campus-software/.

Via Windows win64-x64 Installer at https://cmake.org/download/

Via Windows 64 bit installer at https://git-scm.com/download/win (use all standard options)

https://vcpkg.io/en/getting-started.html

In other words, “assign” or “create”…

https://gitlab.com/libeigen/eigen/-/archive/3.4/eigen-3.4.zip

https://github.com/opencv/opencv/archive/4.5.1.zip

https://www.transmissionzero.co.uk/files/software/development/GLUT/older/freeglut-MSVC-2.8.1-1.mp.zip

https://sourceforge.net/projects/glew/files/glew/2.1.0/glew-2.1.0.zip/download

https://github.com/glfw/glfw/releases/download/3.3.4/glfw-3.3.4.zip

https://pointclouds.org/

http://ceres-solver.org/